# Deep sky astrophotography checklist

Nico Carver

- Bring equipment to flat spot with views of object and Polaris

- Level tripod and add trays/spreaders

- Assemble mount: add pier extension, mount head, and counterweights

- Attach assembled imaging train (scope, camera, guiding) to mount

- Setup power and cables

- Rough polar alignment with polarscope (use iOS app: PS Align Pro)

- Balance Dec then Balance RA

- Turn everything on

- Start Cartes du Ciel and connect to EQMOD

- Start PHD2 and set up guiding camera (ASI290MM-Mini, RAW8)

- Start SGP and set up imaging camera (ASI1600MM-Cool)

- Run QHY Polemaster for fine polar alignment

- Run star alignment procedure in EQMOD

- Slew to bright star with Cartes du Ciel

- Center star with Lum filter and 1 sec. exposures in SGP

- Focus on star with Bahtinov mask and whatever filter I am using first

- Slew to target with SGP

- Center on target

- Plate Solve and slew again

- Center on target

- Run calibration (shift-click) in PHD2

- Start sequence in SGP